### Step 1: Load the image

imageFile = 'image (4).jpeg';  % Replace with your image file path

img = imread(imageFile);  % Read the image

- imageFile stores the name of the image file.

- imread(imageFile) reads the image into the variable img.

### Step 2: Convert to Grayscale if Necessary

if size(img, 3) == 3  % RGB images have 3 color channels

gray_img = rgb2gray(img);  % Convert to grayscale

else

gray_img = img;  % If already grayscale, use as is

end

- The code checks if the image has 3 channels (RGB).

- If yes, it converts the image to grayscale using rgb2gray.

- If already grayscale, it uses the image as is.

### Step 3: Improve Image Quality (Contrast Adjustment and Noise Reduction)

clahe_img = adapthisteq(gray_img, 'ClipLimit', 0.02, 'Distribution', 'rayleigh');

- Adaptive histogram equalization (adapthisteq) enhances local contrast using a clip limit and Rayleigh distribution.

filtered_img = medfilt2(clahe_img, [3 3]);

- medfilt2 applies a median filter to reduce noise, using a 3x3 filter window.

### Step 4: Correct Skewness using Hough Transform

edges_img = edge(filtered_img, 'canny');

- Detects edges in the image using the Canny edge detector.

[H, theta, rho] = hough(edges_img);

peaks = houghpeaks(H, 5);

lines = houghlines(edges_img, theta, rho, peaks);

- Hough transform is applied to detect lines in the image.

- houghpeaks identifies prominent lines.

- houghlines extracts line segments based on detected peaks.

angles = [lines.theta];

if ~isempty(angles)

avg_angle = mean(angles);

rotated_img = imrotate(filtered_img, -avg_angle, 'bilinear', 'crop');

else

rotated_img = filtered_img; % No rotation if no dominant angle detected

end

- Calculates the dominant angle from detected lines.

- Rotates the image by the calculated angle to correct skewness.

figure, imshow(rotated_img);

title('Preprocessed Grayscale Image');

- Displays the preprocessed grayscale image.

### Step 5: Check Contrast Using Standard Deviation

intensity_std = std(double(rotated_img(:)));

- Calculates the standard deviation of pixel intensities.

std_threshold = 20;  % Adjust if needed

disp(['Calculated Standard Deviation: ', num2str(intensity_std)]);

- Compares the intensity standard deviation to a threshold (20).

if intensity_std < std_threshold

disp('Low contrast detected. Applying histogram equalization.');

enhanced_img = histeq(rotated_img);

else

disp('Sufficient contrast detected. Skipping histogram equalization.');

enhanced_img = rotated_img;

end

- If contrast is low, applies histogram equalization to enhance contrast.

### Step 6: Apply Otsu's Thresholding

threshold_value = graythresh(enhanced_img);

bin_img = imbinarize(enhanced_img, threshold_value);

- Computes Otsu's threshold and converts the grayscale image to binary.

bin_img = ~bin_img;

- Inverts the binary image (white background, black text).

### Step 7: Morphological Operations for Noise Reduction

se = strel('disk', 1);

opened_img = imopen(bin_img, se);

processed_img = imclose(opened_img, se);

- Performs morphological opening (removes small noise) and closing (fills small holes) using a disk-shaped structuring element.

### Step 8: Noise Removal

[L, num] = bwlabel(processed_img);

stats = regionprops(L, 'Area');

- Labels connected components in the binary image.

- Extracts the area of each connected component.

area_threshold = 50;  % Components smaller than this will be removed

filtered_img = processed_img;

for i = 1:num

if stats(i).Area < area_threshold

filtered_img(L == i) = 0;

end

end

- Removes small components based on an area threshold.

### Step 9: Character Segmentation

[L, num_components] = bwlabel(filtered_img);

stats = regionprops(L, 'BoundingBox', 'Area');

- Labels connected components for segmentation and extracts bounding boxes.

areas = [stats.Area];

min_size = median(areas) * 0.5;

char_min_size = min_size * 0.2;

disp(['Dynamic min_size: ', num2str(min_size)]);

disp(['Dynamic char_min_size: ', num2str(char_min_size)]);

- Calculates dynamic thresholds for segmentation based on the median area.

### Step 10: Save Segmented Characters

output_folder = 'Seg_Imgs';

if ~exist(output_folder, 'dir')

mkdir(output_folder);

end

- Creates an output folder for saving segmented characters if it doesn't exist.

total_letters = 0;

- Initializes a counter for saved characters.

for i = 1:num_components

if stats(i).Area < min_size

continue;

end

bbox = stats(i).BoundingBox;

line_img = imcrop(filtered_img, bbox);

[L_char, num_chars] = bwlabel(line_img);

char_stats = regionprops(L_char, 'BoundingBox', 'Area');

valid_char_stats = char_stats([char_stats.Area] >= char_min_size);

for j = 1:length(valid_char_stats)

char_bbox = valid_char_stats(j).BoundingBox;

char_img = imcrop(line_img, char_bbox);

total_letters = total_letters + 1;

output_path = fullfile(output_folder, sprintf('char_%d.png', total_letters));

imwrite(char_img, output_path);

end

end

- Iterates through each connected component, crops individual characters, and saves them.

### Step 11: Final Output Messages

disp(['Total number of letters detected and saved: ', num2str(total_letters)]);

disp(' ');

disp(' ');

disp('Saved to Seg_Imgs!');

disp('Please use model to convert to text');

- Displays the number of letters detected and a message indicating where the segmented characters were saved.

### Summary of Process

- Load Image: Reads the image file.

- Preprocessing:

  - Convert to grayscale if needed.

  - Apply contrast enhancement and noise reduction.

  - Skew correction using Hough transform.

- Binarization: Thresholding using Otsu's method.

- Morphological Processing: Noise removal and enhancement.

- Segmentation: Identifies and extracts individual characters.

- Saving Results: Stores segmented characters for further OCR processing.